# UNIT 1: Introduction to Computer Network

Q.1 Define a computer network. Explain its goals, advantages, and need in modern communication systems.

Ans: A computer network is a system in which multiple computers and devices are connected together through communication links to share data, resources, and services. These devices communicate with each other using predefined protocols.

The main goals of a computer network are:

- Resource Sharing
Allows sharing of hardware (printers, scanners), software, and data among multiple users.

- Communication
Enables users to communicate easily through email, messaging, video conferencing, etc.

- Reliability
Data can be stored in multiple locations, ensuring backup and fault tolerance.

- Cost Efficiency
Reduces cost by sharing expensive resources instead of purchasing them individually.

- Scalability
Networks can be expanded easily by adding more devices.

Advantages:

- Fast Communication
Instant transfer of data and messages across the globe.

- Centralized Data Management
Data can be stored and managed at a central location.

- Remote Access
Users can access data and applications from anywhere.

- Collaboration
Multiple users can work on the same project simultaneously.

- Data Backup and Recovery
Ensures data safety through backups.

## Need of Computer Network in Modern Communication Systems:

- Global Connectivity
Enables worldwide communication through the internet.

- Real-Time Communication
Supports video calls, live streaming, and instant messaging.

- Business Operations
Helps organizations in data sharing, decision making, and remote work.

- Education and Research
Enables online learning, virtual classrooms, and information sharing.

Q.2  Differentiate between LAN, MAN, WAN, and PAN with suitable examples.

Ans:

| Parameter | LAN (Local Area Network) | MAN (Metropolitan Area Network) | WAN (Wide Area Network) | PAN (Personal Area Network) |
| --- | --- | --- | --- | --- |
| Ownership | Private | Private or Public | Private or Public | Personal |
| Geographical Area | Small (room, building, campus) | Moderate (city) | Very large (country/continent) | Very small (few meters) |
| Design & Maintenance | Easy | Not easy | Not easy | Very easy |
| Communication Medium | Twisted pair cable (UTP), Wi-Fi | Optical fiber, coaxial cables, wireless | PSTN, satellite links, fiber optics | Bluetooth, USB, Wi-Fi |
| Bandwidth | High | Moderate | High | Low |
| Data Rate (Speed) | High | Moderate | Low to moderate | Low |
| Cost | High | Moderate | Low to moderate | Low |
| Example | College lab network, office network | City-wide cable TV network, ISP network | Internet, bank network across countries | Bluetooth earbuds, smartwatch, mobile hotspot |

Q.3 Explain Ad-hoc networks. List their characteristics and applications.

Ans: A Wireless Ad-hoc Network (MANET – Mobile Ad-hoc Network) is a decentralized wireless network where nodes (devices) communicate directly with each other without relying on any pre-existing infrastructure such as routers, base stations, or access points.

In such networks, each node acts both as a host and a router, forwarding data for other nodes.
The term “ad-hoc” means temporary and formed for a specific purpose, i.e., without any fixed infrastructure.

## Characteristics of Ad-hoc Networks

- Infrastructure-less
No fixed routers or access points are required.

- Dynamic Topology
Nodes can move freely, so network structure changes frequently.

- Multi-hop Communication
Data is forwarded through intermediate nodes.

- Limited Bandwidth
Wireless links have lower capacity compared to wired networks.

- Energy Constraints
Devices usually run on battery power.

## Applications of Ad-hoc Networks

- Military Operations
Communication in battlefields without infrastructure.

- Disaster Recovery
Used during earthquakes, floods where infrastructure is damaged.

- Emergency Services
Firefighters, rescue teams communication.

- Vehicular Ad-hoc Networks (VANETs)
Communication between vehicles.

- Temporary Networks
Conferences, classrooms, or events.

Q.4 Explain Client–Server and Peer-to-Peer architectures. Compare them with advantages and disadvantages.

Ans:

1. Client–Server Architecture

In Client–Server architecture, the network is organized such that a central server provides resources or services, and multiple clients request and use those services.

- The server manages data, security, and processing.

- The client sends requests and receives responses.

2. Peer-to-Peer (P2P) Architecture

In Peer-to-Peer architecture, all devices (peers) are equal and can act as both client and server.

- No central server is required.

- Each node can share and access resources directly.

## Difference:

| Parameter | Client–Server | Peer-to-Peer (P2P) |
| --- | --- | --- |
| Structure | Centralized | Decentralized |
| Control | Controlled by server | No central control |
| Data Management | Centralized | Distributed |
| Security | High | Low |
| Cost | High (server required) | Low |
| Maintenance | Easy (centralized) | Difficult |
| Performance | Better for large systems | Suitable for small networks |

## Advantages & Disadvantages

Client–Server Architecture

Advantages:

- Centralized control and management

- High security

- Easy data backup and recovery

Disadvantages:

- High cost (server setup and maintenance)

- Server failure affects entire network

- Requires skilled administration

Peer-to-Peer Architecture

Advantages:

- Low cost (no dedicated server needed)

- Easy to set up

- No single point of failure

- Suitable for small networks

Disadvantages:

- Low security

- Difficult to manage

- No centralized backup

- Performance decreases with more nodes

Q.5 What is network topology? Explain its importance in network design.

Ans: Network topology refers to the physical or logical arrangement of computers, devices, and communication links in a network. It defines how nodes are connected and how data flows between them.

Common types of topology include bus, star, ring, mesh, and tree topology.

## Importance of Network Topology in Network Design

- Efficient Data Flow
Topology determines how data travels across the network, affecting speed and performance.

- Network Performance
Proper topology selection ensures better bandwidth utilization and reduced congestion.

- Reliability and Fault Tolerance
Some topologies (like mesh) provide multiple paths, reducing the risk of network failure.

- Cost Optimization
Choosing the right topology helps minimize installation and maintenance costs.

- Security
Topology affects how secure the network is and how easily unauthorized access can be controlled.

Q.6 Describe Bus, Ring, and Star topologies with their advantages and disadvantages.

Ans: 1. Bus Topology

Bus topology is a network in which all devices are connected to a single main cable called the backbone.
Data is transmitted in one direction, and both ends of the cable have terminators.

### Advantages

- Cost-effective (less cable required)

- Easy to understand and implement

- Easy to extend by adding cables

### Disadvantages

- If the main cable fails, the entire network fails

- Performance decreases with more devices or heavy traffic

- Limited cable length

2. Ring Topology

In ring topology, each computer is connected to exactly two neighboring devices, forming a circular structure.
Data travels in a sequential manner (usually unidirectional), passing through each node.
A dual ring topology allows data to flow in opposite directions for reliability.

### Advantages

- Performs well under heavy traffic (controlled access using token)

- Equal access for all nodes

- Cheap to install and expand

### Disadvantages

- Failure of one node can affect the whole network

- Difficult to troubleshoot

- Adding/removing nodes disrupts the network

3.Star Topology

In star topology, all devices are connected to a central hub or switch.
Each node has a dedicated connection to the central device.

### Advantages

- Easy to install and manage

- Easy to troubleshoot

- Failure of one node does not affect others

- Easy to expand

### Disadvantages

- Cost of installation is high

- Dependency on central hub (if hub fails, entire network fails)

- Performance depends on the hub capacity

Q. 7 Explain Mesh and Hybrid topologies with suitable examples.

Ans: 1. Mesh Topology

Mesh topology is a network in which each device is connected to every other device through dedicated point-to-point links.
For n devices, the number of links required is n(n−1)/2.

It provides multiple paths for data transmission, making it highly reliable.

### Features

- Fully connected network

- Highly robust

- Not flexible due to complex wiring

### Advantages

- Each connection carries its own data load

- Highly reliable (failure of one link does not affect others)

- Provides high security and privacy

### Disadvantages

- Installation and configuration are difficult

- High cost due to excessive cabling

- Bulk wiring is required

### Example

- Military communication systems

- Internet backbone connections

- Data centers with high reliability requirements

2. Hybrid Topology
Hybrid topology is a combination of two or more different network topologies (such as star, ring, bus).
It is used to combine the advantages of multiple topologies into a single network.

### Features

- Combination of multiple topologies

- Inherits advantages and disadvantages of included topologies

- Flexible and scalable

### Advantages

- Reliable (easy error detection and troubleshooting)

- Scalable (network size can be increased easily)

- Flexible design

- Efficient for large networks

### Disadvantages

- Complex design and management

- High cost

### Example

- Large corporate networks (different departments using different topologies)

- Campus networks (combination of star and bus topology)

- Internet infrastructure

Q.8 Compare Bus, Star, and Ring topologies in terms of cost, performance, and reliability.

Ans:

| Parameter | Bus Topology | Star Topology | Ring Topology |
| --- | --- | --- | --- |
| Cost | Low (uses single cable, no central device) | High (requires hub/switch and more cabling) | Moderate (more cable than bus, no central hub) |
| Performance | Low (decreases with more devices and traffic) | High (good performance, especially with low traffic) | Moderate to High (better than bus, handles traffic efficiently using token passing) |
| Reliability | Low (failure of main cable stops entire network) | High (failure of one node doesn’t affect others, but hub failure affects all) | Moderate (failure of one node can disrupt entire network) |

Q.9 Explain the OSI reference model and describe the functions of each layer.

Ans: The OSI (Open Systems Interconnection) Reference Model is a standard model developed by ISO to enable communication between different computer systems.

It divides the communication process into 7 layers, where each layer performs specific functions and interacts with adjacent layers.

## Features of OSI Model

- Provides a clear structure of network communication

- Shows how hardware and software interact

- Helps in understanding and developing new technologies

## Functions of Each Layer

### 1. Physical Layer (Layer 1)

- Lowest layer of OSI model

- Transmits raw bits over the network

Functions:

- Transmission and reception of raw data

- Defines voltage levels and data rates

- Converts digital data into electrical/optical signals

### 2. Data Link Layer (Layer 2)

- Ensures reliable data transfer between two nodes

Functions:

- Framing of data

- Error detection and correction

- MAC (physical) addressing

### 3. Network Layer (Layer 3)

- Responsible for routing data

Functions:

- Logical addressing (IP address)

- Path selection and routing

- Controls network traffic

### 4. Transport Layer (Layer 4)

- Provides end-to-end communication

Functions:

- Segmentation and reassembly

- Error control and flow control

- Ensures reliable or fast delivery (TCP/UDP)

### 5. Session Layer (Layer 5)

- Manages communication sessions

Functions:

- Establishes, maintains, and terminates sessions

- Synchronization and checkpoints

- Prevents data loss during transmission

### 6. Presentation Layer (Layer 6)

- Acts as a translator

Functions:

- Data translation (format conversion)

- Encryption and decryption

- Data compression

### 7. Application Layer (Layer 7)

- Closest to the user

Functions:

- Provides network services to applications

- File transfer, email, web services

- Interface between user and network

Q. 10 Explain the TCP/IP model and compare it with the OSI model, including design issues like addressing, error control, and flow control.

Ans: The TCP/IP (Transmission Control Protocol / Internet Protocol) model is a practical networking model developed by DARPA for communication over the Internet.
It consists of 4 layers and is widely used in real-world networking.

## Layers of TCP/IP Model

### 1. Host-to-Network (Network Access Layer)

- Lowest layer

- Deals with physical transmission of data

Functions:

- Interface between host and network

- Handles hardware addressing and data transmission

- Uses protocols like Ethernet, Wi-Fi

### 2. Internet Layer

- Responsible for packet delivery

Functions:

- Logical addressing (IP address)

- Routing of packets

- Packet forwarding

### 3. Transport Layer

- Provides end-to-end communication

Functions:

- Segmentation and reassembly

- Flow control

- Error control

### 4. Application Layer

- Combines OSI’s top three layers

Functions:

- Provides services like email, file transfer, web

- Protocols: HTTP, FTP, SMTP, DNS, TELNET

Comparison:

| Parameter | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Number of Layers | 7 layers | 4 layers |
| Nature | Conceptual (reference model) | Practical (implementation model) |
| Development | Developed by ISO | Developed by DARPA |
| Approach | Vertical approach | Horizontal approach |
| Layer Structure | Separate Session & Presentation layers | These are combined in Application layer |
| Protocol Dependency | Protocol independent | Protocol dependent |
| Usage | Used for teaching/design | Used in real-world Internet |

## Design Issues in TCP/IP & OSI Models

### 1. Addressing

- Identifies sender and receiver

- OSI: Uses logical (IP) and physical (MAC) addressing

- TCP/IP: Uses IP addresses (Internet layer) and port numbers (Transport layer)

### 2. Error Control

- Ensures reliable data transmission

- OSI:

  - Data Link Layer → detects errors

  - Transport Layer → corrects errors

- TCP/IP:

  - TCP provides error detection and recovery

  - UDP does not provide error correction

### 3. Flow Control

- Controls the rate of data transmission

- OSI: Managed mainly by Transport Layer

- TCP/IP:

  - TCP uses sliding window mechanism

  - Prevents receiver overload

Q.11 Define transmission media. Classify transmission media into guided and unguided media with examples.

Ans: Transmission media refers to the physical path or channel through which data is transmitted from sender to receiver in a network.
It carries signals in the form of electrical, optical, or electromagnetic waves.

Classification of Transmission Media:

- Guided Media (Wired Communication)

Guided media are those in which signals are transmitted through a physical path (cables or wires).

Types & Examples:

- Twisted Pair Cable (UTP/STP)

  - Used in LANs and telephone lines

- Coaxial Cable

  - Used in cable TV and broadband

- Optical Fiber Cable

  - Uses light signals, high speed

  - Used in internet backbone

Advantages:

- High security

- Less interference

- Reliable communication

Disadvantages:

- Limited mobility

- Installation cost can be high

- Unguided Media (Wireless Communication)

Unguided media transmit data through air or space using electromagnetic waves, without physical connection.

Types & Examples:

- Radio Waves

  - Used in radio, TV broadcasting

- Microwaves

  - Used in mobile communication, Wi-Fi

- Infrared

  - Used in remote controls

- Satellite Communication

  - Used in GPS, TV broadcasting

Advantages:

- Supports mobility

- Easy installation

- Wide coverage

Disadvantages:

- More interference

- Less secure

- Affected by environmental conditions

Q.12 Differentiate between Half-Duplex and Full-Duplex transmission modes with respect to direction of data flow, efficiency, and examples.

Ans:

| Parameter | Half-Duplex | Full-Duplex |
| --- | --- | --- |
| Direction of Data Flow | Data flows in both directions but not at the same time | Data flows in both directions simultaneously |
| Efficiency | Less efficient (time is wasted while switching direction) | More efficient (continuous simultaneous communication) |
| Communication Type | One device transmits at a time | Both devices can transmit and receive at the same time |
| Channel Usage | Single channel shared alternately | Channel is used simultaneously (often separate paths) |
| Examples | Walkie-talkie, CB radio | Telephone, video calls |

Q.13 Explain the working of a Hub and a Repeater. How do they differ in terms of functionality and OSI layer?

Ans:  HUB

A Hub is a basic networking device used to connect multiple computers in a LAN.
It operates at the Physical Layer (Layer 1) of the OSI model.

### Working:

- When a hub receives a data signal from one device, it broadcasts (copies) the signal to all other connected devices.

- It does not filter or check addresses.

- Every device receives the data, but only the intended device accepts it.

REPEATER

A Repeater is a device used to extend the range of a network by regenerating signals.
It also works at the Physical Layer (Layer 1).

### Working:

- It receives a weak or distorted signal

- Cleans, amplifies, and regenerates the signal

- Transmits the refreshed signal to the next segment

Difference between Hub and Repeater

| Parameter | Hub | Repeater |
| --- | --- | --- |
| Function | Connects multiple devices and broadcasts data | Regenerates and strengthens signals |
| Purpose | Used to create a network | Used to extend network distance |
| Data Handling | Sends data to all ports (broadcast) | Does not send data to devices, only forwards signal |
| Number of Ports | Multiple ports | Usually two ports |
| OSI Layer | Physical Layer (Layer 1) | Physical Layer (Layer 1) |

Q.14 What is a Bridge? Explain its role in a network and how it reduces network traffic.

Ans: A Bridge is a networking device that connects two or more LAN segments and operates at the Data Link Layer (Layer 2) of the OSI model.
It is used to filter and forward data frames based on MAC (physical) addresses.

## Working of a Bridge

- A bridge receives a frame from one network segment

- It checks the destination MAC address

- If the destination is in the same segment, it blocks the frame

- If the destination is in a different segment, it forwards the frame

## Role of Bridge in a Network

- Segmentation of Network
Divides a large network into smaller segments

- Traffic Filtering
Allows only necessary data to pass between segments

- Improved Performance
Reduces congestion and improves efficiency

- Extension of Network
Connects multiple LANs to form a larger network

## How Bridge Reduces Network Traffic

- Selective Forwarding
Only forwards frames meant for another segment

- Collision Reduction
Creates separate collision domains, reducing collisions

- Filtering Unnecessary Data
Prevents local traffic from spreading to other segments

Q.15 Compare Hub, Switch, and Router based on OSI layer, data handling, intelligence, and usage.

Ans:

| Parameter | Hub | Switch | Router |
| --- | --- | --- | --- |
| OSI Layer | Physical Layer (Layer 1) | Data Link Layer (Layer 2) | Network Layer (Layer 3) |
| Data Handling | Broadcasts data to all devices (no filtering) | Forwards data only to the intended device using MAC address | Routes data between networks using IP address |
| Intelligence | No intelligence (dumb device) | Moderate intelligence (learns MAC addresses) | High intelligence (uses routing tables and algorithms) |
| Usage | Used to connect devices in small LAN (rarely used now) | Used in LANs for efficient communication | Used to connect different networks (e.g., LAN to Internet) |

Q.16 Explain the working and functions of Hub, Repeater, and Bridge. Compare them.

Ans:  1. HUB

### Working

- A hub receives data from one device

- It broadcasts the data to all connected devices

- It does not check addresses or filter data

### Functions

- Connects multiple devices in a LAN

- Acts as a central connection point

- Transmits signals to all ports

2. REPEATER

### Working

- Receives weak or distorted signals

- Regenerates and amplifies the signal

- Sends the refreshed signal forward

### Functions

- Extends network distance

- Restores signal strength

- Reduces signal loss

3. BRIDGE

### Working

- Receives a frame from one segment

- Checks the MAC address

- Forwards or blocks the frame based on destination

### Functions

- Connects multiple LAN segments

- Filters traffic

- Reduces network congestion

Comparison of Hub, Repeater, and Bridge

| Parameter | Hub | Repeater | Bridge |
| --- | --- | --- | --- |
| OSI Layer | Physical Layer (L1) | Physical Layer (L1) | Data Link Layer (L2) |
| Function | Connects devices and broadcasts data | Regenerates signal | Connects segments and filters data |
| Data Handling | Broadcasts to all devices | Does not handle data, only signal | Forwards selectively using MAC address |
| Intelligence | No intelligence | No intelligence | Intelligent (uses MAC table) |
| Purpose | Create LAN | Extend network distance | Reduce traffic and segment network |

Q.17 Define Router and Gateway. Explain their functions and highlight the differences between them.

Ans: 1. ROUTER

A Router is a networking device that connects multiple networks and forwards data packets between them using IP (logical) addresses.
It operates at the Network Layer (Layer 3) of the OSI model.

### Functions:

- Packet Routing
Determines the best path for data using routing algorithms

- Logical Addressing
Uses IP addresses to identify source and destination

- Network Connectivity
Connects different networks (e.g., LAN to Internet)

- Traffic Management
Controls and directs data traffic efficiently

- Security Features
Provides basic firewall and filtering

2.GATEWAY

A Gateway is a network device that acts as a protocol converter, enabling communication between different types of networks that use different protocols.
It can operate at multiple layers (often higher layers) of the OSI model.

### Functions:

- Protocol Conversion
Converts data formats between different protocols

- Communication Between Dissimilar Networks
Enables interaction between networks with different architectures

- Data Translation
Translates data, formats, and encoding

- Security and Filtering
Can act as firewall or proxy

Difference between Router and Gateway

| Parameter | Router | Gateway |
| --- | --- | --- |
| Definition | Connects networks and routes packets | Connects dissimilar networks by converting protocols |
| OSI Layer | Network Layer (Layer 3) | Works across multiple layers (often higher layers) |
| Function | Path determination and packet forwarding | Protocol conversion and data translation |
| Communication Type | Between similar networks | Between different networks |
| Complexity | Less complex | More complex |
| Example | Home router connecting to Internet | Email gateway, VoIP gateway |

Q.18 Explain guided transmission media. Describe twisted pair cable, coaxial cable, and optical fiber with their advantages and disadvantages.

Ans: Guided transmission media are communication channels in which data signals travel through a physical path (cables or wires) from sender to receiver.
They provide a directed and secure transmission of data.

Types of Guided Media:

## 1. Twisted Pair Cable

### Description

- Consists of two insulated copper wires twisted together

- Twisting reduces electromagnetic interference

- Types: UTP (Unshielded) and STP (Shielded)

### Advantages

- Low cost

- Easy to install and maintain

- Flexible and lightweight

### Disadvantages

- Limited bandwidth

- Prone to noise and interference (especially UTP)

- Short distance communication

## 2. Coaxial Cable

### Description

- Has a central conductor, insulating layer, metallic shield, and outer covering

- Provides better shielding than twisted pair

### Advantages

- Higher bandwidth than twisted pair

- Better noise immunity

- More reliable

### Disadvantages

- More expensive than twisted pair

- Bulky and less flexible

- Installation is complex

## 3. Optical Fiber Cable

### Description

- Uses light signals to transmit data through glass or plastic fibers

- Based on total internal reflection

### Advantages

- Very high bandwidth

- Extremely fast data transmission

- Highly secure

### Disadvantages

- Expensive

- Difficult to install and maintain

- Fragile compared to copper cables

Q.19 Compare optical fiber and twisted pair cable for below terms- Bandwidth, Cost, Attenuation, Applications.

Ans:

| Parameter | Optical Fiber Cable | Twisted Pair Cable |
| --- | --- | --- |
| Bandwidth | Very high (supports very high data rates) | Low to moderate |
| Cost | Expensive (installation and maintenance) | Low (economical) |
| Attenuation | Very low (signal loss is minimal) | High (more signal loss over distance) |
| Applications | Internet backbone, long-distance communication, high-speed networks | Telephone lines, LANs, small networks |

Q.20 What is unguided transmission media? Explain radio waves, microwaves, and infrared waves with suitable use cases.

Ans: Unguided transmission media (wireless media) are communication channels in which data is transmitted through air or free space using electromagnetic waves, without any physical connection between sender and receiver.

## Types of Unguided Media

## 1. Radio Waves

### Description

- Frequency range: 3 kHz to 1 GHz

- Travel in all directions (omnidirectional)

- Can penetrate walls and obstacles

### Use Cases

- Radio broadcasting (FM/AM)

- Television transmission

- Wireless LANs (Wi-Fi)

## 2. Microwaves

### Description

- Frequency range: 1 GHz to 300 GHz

- Travel in straight line (line-of-sight communication)

- Require antennas or satellite links

### Use Cases

- Mobile communication (cellular networks)

- Satellite communication

- Point-to-point communication links

## 3. Infrared Waves

### Description

- Short-range communication

- Cannot penetrate walls

- Requires line-of-sight

### Use Cases

- TV remote controls

- Wireless mouse/keyboard

- Short-range device communication

UNIT 2: Data Link Layers

Q.1 Give brief about design issues in DLL

Ans: The Data Link Layer (DLL) is responsible for node-to-node (hop-to-hop) delivery of data.
It ensures that data is transferred reliably and efficiently over a physical link.

## Design Issues in Data Link Layer

### 1. Providing Services to Network Layer

- DLL provides a well-defined interface to the Network Layer

- It hides errors and transmission details

Types of Services:

- Unacknowledged connectionless service

- Acknowledged connectionless service

- Acknowledged connection-oriented service

### 2. Framing

- DLL converts raw bits into frames

- Frames are units of data transmission

Purpose:

- To mark the beginning and end of data

- To ensure proper synchronization

### 3. Error Control

- Ensures error-free data transmission

Methods:

- Error detection (Parity, CRC, Checksum)

- Error correction (ARQ, Hamming code)

### 4. Flow Control

- Controls the rate of data transmission

- Prevents fast sender from overwhelming slow receiver

Techniques:

- Stop-and-Wait

- Sliding Window

### 5. Access Control (MAC – Medium Access Control)

- Determines which device can use the channel at a time

Examples:

- CSMA/CD

- CSMA/CA

- Token passing

Q.2 What are Framing, Error control and Flow control

Ans: 1. Framing

Framing is the process of dividing a continuous stream of bits into manageable units called frames for transmission over a network.

### Purpose

- To mark the start and end of data

- To ensure synchronization between sender and receiver

- To enable error detection

### Methods of Framing

- Byte Count Method

- Byte Stuffing (Character Stuffing)

- Bit Stuffing

## 2. Error Control

Error control is the mechanism used to detect and correct errors that occur during data transmission.

### Purpose

- To ensure reliable communication

- To detect corrupted data and correct it

### Techniques

- Error Detection

  - Parity Check

  - CRC (Cyclic Redundancy Check)

  - Checksum

- Error Correction

  - Hamming Code

  - ARQ (Automatic Repeat Request)

## 3. Flow Control

Flow control is a technique used to control the rate of data transmission between sender and receiver.

### Purpose

- To prevent data loss due to buffer overflow

- To match sender speed with receiver capacity

### Techniques

- Stop-and-Wait Protocol

- Sliding Window Protocol

Q.3 Give short note on CSMA/CA, CSMA/CD.

Ans:

## 1. CSMA/CD (Carrier Sense Multiple Access with Collision Detection)

CSMA/CD is a network protocol used in wired networks (Ethernet) to detect collisions during data transmission.

### Working

- A station listens to the channel before transmitting

- If the channel is idle, it starts transmission

- If a collision is detected, it immediately stops transmission

- Waits for a random time (backoff time)

- Retransmits the data

### Key Features

- Detects collision after it occurs

- Used in half-duplex Ethernet

- Improves efficiency compared to ALOHA

## 2. CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance)

CSMA/CA is a protocol used in wireless networks (Wi-Fi) to avoid collisions before they occur.

### Working

- Station senses the channel

- If busy → waits until free

- Waits for a fixed time (DIFS – Distributed Inter Frame Space)

### Key Features

- Avoids collision instead of detecting

- Used in wireless networks (IEEE 802.11)

- Uses acknowledgement (ACK) mechanism

Q.4 Explain Stop and Wait ARQ Protocol.

Ans: Stop-and-Wait ARQ (Automatic Repeat reQuest) is an error control protocol in which the sender transmits one frame at a time and waits for an acknowledgment (ACK) from the receiver before sending the next frame.

## Working of Stop-and-Wait ARQ

- The sender sends one data frame to the receiver

- The sender starts a timer after sending the frame

- The receiver checks the frame for errors

- If the frame is correct, it sends an ACK

- If the frame is corrupted or lost, no ACK is sent

- If ACK is received → sender sends the next frame

- If ACK is not received within time → sender retransmits the frame

## Key Features

- Uses sequence numbers (0 and 1) to avoid duplication

- Only one frame is in transit at a time

- Simple and easy to implement

## Advantages

- Simple and easy to understand

- Ensures reliable communication

- Requires less memory

## Disadvantages

- Low efficiency (channel remains idle while waiting for ACK)

- Not suitable for high-speed networks

- High delay due to waiting time

Q.6 What is sliding window protocol? How it works?

Ans: The Sliding Window Protocol is a flow control protocol that allows the sender to transmit multiple frames before receiving acknowledgments, improving efficiency.

## Working

- The sender maintains a window of size N (number of frames it can send without ACK)

- Frames are assigned sequence numbers

- The sender can send all frames within the window

- The receiver sends ACKs for received frames

- As ACKs are received, the window slides forward

- Lost frames are retransmitted (depends on protocol type)

## Types of Sliding Window Protocol

- Go-Back-N ARQ

  - Sender retransmits all frames from the lost frame onward

- Selective Repeat ARQ

  - Only the lost or corrupted frames are retransmitted

## Advantages

- High efficiency (multiple frames sent together)

- Better utilization of bandwidth

- Suitable for high-speed networks

## Disadvantages

- More complex than Stop-and-Wait

- Requires buffer memory

- Sequence number management needed

Q.7 Explain CSMA in detail.

Ans: CSMA (Carrier Sense Multiple Access) is a network protocol in which a device senses the channel before transmitting to reduce collisions.

## Working

- A station checks whether the channel is idle or busy

- If idle → transmit immediately

- If busy → wait and try again

- Collisions may still occur due to propagation delay

## Types of CSMA

### 1. 1-Persistent CSMA

- If channel is idle → transmit immediately

- If busy → continuously sense until idle

### 2. Non-Persistent CSMA

- If channel is busy → wait for random time

- Then sense again

### 3. p-Persistent CSMA

- Used in slotted channels

- Transmit with probability p

- Wait with probability 1-p

## Advantages

- Reduces collisions compared to ALOHA

- Improves network efficiency

- Simple and effective

## Disadvantages

- Collisions still possible

- Performance decreases under heavy load

- Delay due to sensing and waiting

Q.8 Differentiate between Pure and Slotted ALOHA

Ans:

| Parameter | Pure ALOHA | Slotted ALOHA |
| --- | --- | --- |
| Transmission Time | Data can be sent at any time | Data is sent only at the beginning of time slots |
| Synchronization | No synchronization required | Requires synchronization between stations |
| Collision Probability | High (frames can overlap at any time) | Lower (collisions only at slot boundaries) |
| Efficiency (Throughput) | Low (~18.4%) | Higher (~36.8%) |
| Vulnerable Period | 2 × frame time | 1 × frame time |
| Performance | Poor due to frequent collisions | Better due to reduced collisions |

Q.10 Draw flowchart of CSMA/CA.

Ans:

Q11. Explain Selective repeat ARQ in details.

Ans: Selective Repeat ARQ is an error control protocol in which the sender retransmits only the frames that are lost or corrupted, instead of retransmitting all frames.

## Working

- Sender transmits multiple frames within a window size

- Each frame has a sequence number

- Receiver accepts frames (even out of order) and stores them in buffer

- Receiver sends ACK for each correctly received frame

- If a frame is missing → receiver sends NAK (Negative ACK)

- Sender retransmits only the missing frames

- Once all frames are received → data is delivered in order

## Features

- Uses sliding window mechanism

- Allows out-of-order reception

- Requires buffer at receiver side

## Advantages

- High efficiency

- Only lost frames are retransmitted

- Better bandwidth utilization

## Disadvantages

- Complex implementation

- Requires more memory (buffering)

- Sequence number management is needed

Q.15 What is the need for framing? What are the different techniques of framing? Explain any two.

Ans: Framing is the process of dividing a continuous stream of bits into small manageable units called frames in the Data Link Layer.

### Need / Importance

- Delimitation – Identifies the start and end of a frame

- Synchronization – Helps sender and receiver stay synchronized

- Error Detection – Allows addition of error detection bits (CRC, parity)

- Flow Control – Enables proper control of data transmission

- Addressing – Frames can include source and destination addresses

## Techniques of Framing

- Byte Count (Character Count) Method

- Byte Stuffing (Character Stuffing)

- Bit Stuffing

- Physical Layer Coding Violations

## Explain Any Two Techniques

## 1. Byte Stuffing (Character Stuffing)

Special characters (called flag bytes) are added at the beginning and end of a frame.

### Working

- A special flag byte marks frame boundaries

- If the same byte appears in data → an escape character is inserted

- Receiver removes the extra inserted characters

### Example

- Flag = 0x7E

- If data contains 0x7E, it is replaced with escape sequence

### Advantage

- Clearly identifies frame boundaries

### Disadvantage

- Increases overhead due to extra bytes

## 2. Bit Stuffing

In bit stuffing, a 0 bit is inserted after every sequence of five consecutive 1s in the data.

### Working

- Frames start and end with a special bit pattern (e.g., 01111110)

- To avoid confusion, a 0 is inserted after five 1s

- Receiver removes the stuffed 0 bits

### Example

- Data: 11111 → becomes 111110

### Advantage

- Works at bit level, efficient for binary data

### Disadvantage

- Slight increase in data size

Q.16 Explain the Go-Back-N sliding window protocol with the help of a suitable example.

Ans: Go-Back-N ARQ is a sliding window protocol in which the sender can send multiple frames without waiting for acknowledgment, but if an error occurs, it retransmits the erroneous frame and all subsequent frames.

## Working

- Sender maintains a window size = N

- It can send N frames continuously without waiting for ACK

- Each frame has a sequence number

- Receiver sends cumulative ACKs (acknowledges last correctly received frame)

- If a frame is lost or damaged:

  - Receiver discards that frame and all following frames

  - Sends NAK or no ACK

- Sender, after timeout, goes back to the lost frame and retransmits all frames from that point

## Example

Let:

- Window size = 4

- Frames = 0, 1, 2, 3, 4, 5, 6

### Step-by-step:

- Sender sends frames: 0, 1, 2, 3

- Receiver receives and sends ACK for each

- Sender sends next frames: 4, 5, 6

- Suppose frame 5 is lost

- Receiver receives frame 6 but discards it (since 5 is missing)

- Receiver sends ACK for last correct frame (ACK 4)

- Sender times out and retransmits: 5 and 6

## Features

- Uses cumulative acknowledgment

- Receiver does not store out-of-order frames

- Simple compared to Selective Repeat

## Advantages

- Easy to implement

- Efficient compared to Stop-and-Wait

- Better bandwidth utilization

## Disadvantages

- Wastage of bandwidth (retransmits multiple frames)

- Less efficient than Selective Repeat

- Not suitable for highly noisy channels

Q.18 State the differences between static channel allocation and dynamic channel allocation. Give two examples of each.

Ans:

| Parameter | Static Channel Allocation | Dynamic Channel Allocation |
| --- | --- | --- |
| Definition | Channel is permanently divided and assigned to users | Channel is allocated on demand when needed |
| Usage of Channel | Fixed, even if user is idle | Used only when user has data to send |
| Efficiency | Low (wastage of bandwidth when idle) | High (better utilization of bandwidth) |
| Flexibility | Not flexible | Highly flexible |
| Delay | No delay in allocation | May have delay due to allocation process |
| Complexity | Simple | More complex |
| Collision | No collision (pre-assigned) | Collision possible (needs control protocols) |

## Examples

### Static Channel Allocation

- FDMA (Frequency Division Multiple Access)

- TDMA (Time Division Multiple Access)

### Dynamic Channel Allocation

- CSMA/CD (Carrier Sense Multiple Access with Collision Detection)

- CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance)

Q.19 Explain bit stuffing and character stuffing framing method in DLL.

Ans: 1. Character Stuffing (Byte Stuffing)

Character stuffing is a framing method in which special characters (flag bytes) are used to mark the beginning and end of a frame.

### Working

- A special flag byte (e.g., 0x7E) is added at the start and end of the frame

- If the same flag byte appears inside the data → an escape character (0x7D) is inserted before it

- At the receiver side → escape characters are removed to get original data

### Example

- Original Data: A 0x7E B

- After Stuffing: A 0x7D 0x5E B

### Advantages

- Clearly identifies frame boundaries

- Easy to implement

### Disadvantages

- Increases overhead due to extra bytes

- Less efficient for binary data

## 2. Bit Stuffing

Bit stuffing is a method in which a 0 bit is inserted after every sequence of five consecutive 1s in the data.

### Working

- Frames begin and end with a special bit pattern (e.g., 01111110)

- Whenever the sender encounters five consecutive 1s, it inserts a 0 bit

- Receiver removes the inserted 0 bits (de-stuffing)

### Example

- Original Data: 11111

- After Stuffing: 111110

### Advantages

- Efficient for binary transmission

- Ensures proper frame synchronization

### Disadvantages

- Slight increase in data size

- Additional processing required

Q.20 Explain 1-persistent and non-persistent CSMA protocol.

Ans: 1-Persistent CSMA

In 1-persistent CSMA, a station transmits immediately (with probability = 1) when the channel becomes idle.

### Working

- Station senses the channel

- If channel is idle → transmit immediately

- If channel is busy → continuously sense

- As soon as channel becomes free → transmit

### Characteristics

- Always ready to transmit

- High probability of collision

### Advantages

- Simple and fast transmission

- No unnecessary waiting

### Disadvantages

- High chance of collision (many stations transmit at once)

- Network congestion under heavy load

## 2. Non-Persistent CSMA

In non-persistent CSMA, a station waits for a random time if the channel is busy instead of continuously sensing it.

### Working

- Station senses the channel

- If idle → transmit immediately

- If busy → wait for random time

- After waiting → sense channel again and transmit

### Characteristics

- Reduces collision probability

- Introduces delay

### Advantages

- Fewer collisions

- Better performance under heavy load

### Disadvantages

- Increased delay

- Possible idle time wastage

Q.21 Explain CSMA/CD protocol in detail.

Ans: CSMA/CD (Carrier Sense Multiple Access with Collision Detection) is a medium access control protocol used in wired Ethernet networks to control data transmission and detect collisions.

## Working of CSMA/CD

- Carrier Sensing

  - A station first listens to the channel to check if it is idle or busy

- Transmission

  - If the channel is idle → transmit data

  - If busy → wait until it becomes idle

- Collision Detection

  - While transmitting, the station continues to monitor the channel

  - If a collision occurs, it is detected immediately

- Jam Signal

  - The station sends a jam signal to inform all stations about the collision

- Backoff Algorithm

  - Stations wait for a random time (backoff time) before retransmission

- Retransmission

  - After waiting, the station again senses the channel and retransmits

## Key Features

- Works on half-duplex Ethernet

- Uses binary exponential backoff

- Reduces collision wastage compared to ALOHA

## Advantages

- Simple and efficient for wired networks

- Reduces collision impact

- Improves network performance

## Disadvantages

- Not suitable for wireless networks

- Performance decreases under heavy traffic

- Collisions still occur

Q.22 List the functions of data link layers? Define framing, flow control, Error control and Media access control.

## Ans:  The Data Link Layer is responsible for node-to-node delivery of data. Its main functions are:

- Framing
Divides the bit stream into frames for transmission

- Error Control
Detects and corrects errors to ensure reliable communication

- Flow Control
Controls the rate of data transmission between sender and receiver

- Media Access Control (MAC)
Decides which device can access the shared communication medium

- Physical Addressing
Adds MAC address to identify sender and receiver

- Link Management
Establishes and terminates links between nodes

### 1. Framing

Framing is the process of dividing a continuous stream of bits into frames, adding headers and trailers to mark the beginning and end of data.

### 2. Flow Control

Flow control is a technique used to control the rate of data transmission so that a fast sender does not overwhelm a slow receiver.

### 3. Error Control

Error control ensures accurate data transmission by detecting and correcting errors using techniques like CRC, parity, and ARQ.

### 4. Media Access Control (MAC)

MAC is a mechanism that determines how devices share and access a common communication channel, preventing collisions.

Q.23 Explain with diagram any four functions of the Data Link Layer.

Ans:

## 1. Framing

- Converts a stream of bits into frames

- Adds header and trailer to identify boundaries

Purpose:
Ensures proper synchronization and data organization

## 2. Error Control

- Detects and corrects errors in transmitted data

Techniques:

- Parity check

- CRC

- ARQ protocols

Purpose:
Ensures reliable communication

## 3. Flow Control

- Controls the rate of data transmission

Techniques:

- Stop-and-Wait

- Sliding Window

Purpose:
Prevents receiver overload

## 4. Media Access Control (MAC)

- Decides which device can access the channel

Examples:

- CSMA/CD

- CSMA/CA

- Token passing

Purpose:
Avoids collisions and ensures fair usage

Q.25 Explain the Stop and Wait Protocol in Noiseless Channel with detailed schematic.

ANs:

The Stop-and-Wait Protocol is a simple flow control protocol in which the sender sends one frame at a time and waits for acknowledgment before sending the next frame.
In a noiseless channel, no frames are lost or corrupted.

## Working

- The sender sends one frame to the receiver

- The receiver receives the frame correctly (no errors)

- The receiver sends an ACK (acknowledgment)

- After receiving ACK, the sender sends the next frame

- This process continues until all frames are transmitted

## Characteristics

- Only one frame in transit at a time

- No errors or retransmissions

- Uses acknowledgment for each frame

## Advantages

- Simple and easy to implement

- No data loss in noiseless channel

- Requires less memory

## Disadvantages

- Low efficiency (sender waits after each frame)

- Wastage of bandwidth

- Not suitable for high-speed networks

UNIT 3: Network Layer

Q.1 Define Network Layer and list its functions.

Ans: The Network Layer is the third layer (Layer 3) of the OSI model responsible for source-to-destination delivery of packets across multiple networks.
It ensures that data is transferred from the sender to the receiver even if they are on different networks.

## Functions of Network Layer

- Logical Addressing

  - Assigns IP addresses to devices

  - Identifies source and destination uniquely

- Routing (Path Determination)

  - Determines the best path for data transmission

  - Uses routing algorithms

- Packet Forwarding

  - Moves packets from one router to another

  - Ensures delivery to correct destination

- Internetworking

  - Connects different networks (LANs, WANs)

  - Enables communication across networks

- Fragmentation and Reassembly

  - Breaks large packets into smaller fragments

  - Reassembles them at destination

- Congestion Control

  - Controls network traffic

  - Prevents overload of network

- Error Handling (Limited)

  - Detects packet loss and reports errors (e.g., ICMP)

Q.2 List IPv4 address classes and their ranges.

Ans:  IPv4 addresses are 32-bit addresses divided into different classes to support networks of different sizes.

| Class | First Octet Range | Starting Bits | Default Subnet Mask | Usage |
| --- | --- | --- | --- | --- |
| Class A | 1 – 126 | 0 | 255.0.0.0 | Large networks |
| Class B | 128 – 191 | 10 | 255.255.0.0 | Medium networks |
| Class C | 192 – 223 | 110 | 255.255.255.0 | Small networks |
| Class D | 224 – 239 | 1110 | Not defined | Multicasting |
| Class E | 240 – 255 | 1111 | Not defined | Experimental |

Q.3 What is NAT? State its purpose.

Ans: NAT (Network Address Translation) is a technique used in networking where a private IP address is translated into a public IP address (or vice versa) by a router while data is being transmitted.

## Purpose of NAT

- Conservation of IP Addresses

  - Allows multiple devices to share a single public IP address

  - Helps in reducing IPv4 address exhaustion

- Security Enhancement

  - Hides internal (private) IP addresses from external networks

  - Protects the internal network from direct access

- Private Network Usage

  - Enables use of private IP addresses within an organization

  - No need for globally unique IPs for every device

- Flexibility in Network Design

  - Internal network structure can be changed without affecting external communication

Types of NAT

- Static NAT – One-to-one mapping

- Dynamic NAT – Pool of public IPs

- PAT (Port Address Translation) – Many-to-one mapping (most common)

Q.4 Explain classful and classless addressing.

## Ans: 1. Classful Addressing

Classful addressing is an IPv4 addressing method in which IP addresses are divided into fixed classes (A, B, C, D, E) based on predefined boundaries.

### Features

- Fixed network and host portions

- Uses default subnet masks

- No flexibility in allocation

### Classes Overview

- Class A → Large networks

- Class B → Medium networks

- Class C → Small networks

### Limitations

- Wastage of IP addresses

- Not efficient for modern networks

- No support for subnetting flexibility

## 2. Classless Addressing (CIDR – Classless Inter-Domain Routing)

Classless addressing allows IP addresses to be allocated without fixed classes, using a prefix length (subnet mask).

### Representation

- Written as: IP address / prefix length
Example: 192.168.1.0/24

### Features

- Flexible allocation of IP addresses

- Efficient use of address space

- Supports subnetting and supernetting

### Advantages

- Reduces IP address wastage

- Improves routing efficiency

- Supports modern internet requirements

Q5. Describe static and dynamic IP addressing.

## Ans: 1. Static IP Addressing

A Static IP address is an IP address that is manually assigned to a device and remains constant over time.

### Features

- Fixed and does not change

- Assigned by network administrator

- Requires manual configuration

### Advantages

- Stable and reliable connection

- Suitable for servers and network devices

- Easy remote access

### Disadvantages

- Time-consuming to configure

- Not scalable for large networks

- Risk of IP conflicts if misconfigured

### Examples

- Web servers

- DNS servers

- Network printers

## 2. Dynamic IP Addressing

A Dynamic IP address is an IP address that is automatically assigned to a device by a DHCP (Dynamic Host Configuration Protocol) server.

### Features

- IP address changes periodically

- Assigned automatically

- No manual configuration needed

### Advantages

- Easy to manage

- Efficient use of IP addresses

- Suitable for large networks

### Disadvantages

- IP address may change

- Less suitable for hosting servers

- Dependency on DHCP server

### Examples

- Home networks

- Mobile devices

- Office networks

Q6. Explain subnetting with example.

Ans:  Subnetting is the process of dividing a large network into smaller sub-networks (subnets) to improve network efficiency, security, and management.

## Need for Subnetting

- Reduces network congestion

- Improves performance

- Enhances security

- Efficient use of IP addresses

## Concept

An IP address has two parts:

- Network part

- Host part

## Example

### Given:

IP Address: 192.168.1.0/24
(Default subnet mask: 255.255.255.0)

### Step 1: Borrow Bits

Borrow 2 bits from host part

New subnet mask = /26
👉 255.255.255.192

### Step 2: Calculate Number of Subnets

Number of subnets = 2² = 4

### Step 3: Calculate Hosts per Subnet

Remaining bits = 6
Hosts per subnet = 2⁶ − 2 = 62

Step 4: Subnet Ranges

| Subnet | Network Address | Host Range | Broadcast Address |
| --- | --- | --- | --- |
| Subnet 1 | 192.168.1.0 | 192.168.1.1 – 62 | 192.168.1.63 |
| Subnet 2 | 192.168.1.64 | 192.168.1.65 – 126 | 192.168.1.127 |
| Subnet 3 | 192.168.1.128 | 192.168.1.129 – 190 | 192.168.1.191 |
| Subnet 4 | 192.168.1.192 | 192.168.1.193 – 254 | 192.168.1.255 |

## Advantages of Subnetting

- Efficient IP address utilization

- Improved network performance

- Better security and management

Q8. Explain NAT translation with example.

Ans: NAT (Network Address Translation) is a technique used to convert private IP addresses into public IP addresses for communication over the Internet.

## Working of NAT

- Internal device sends request with private IP

- Router replaces it with public IP

- Maintains a translation table

- Response from internet is mapped back to original device

## Example

### Before NAT

- Device IP: 192.168.1.2

- Destination: 8.8.8.8

### After NAT

- Router replaces source IP → 203.0.113.1 (public IP)

### NAT Table

| Private IP | Public IP |
| --- | --- |
| 192.168.1.2 | 203.0.113.1 |

### Process

- Request goes:
192.168.1.2 → 8.8.8.8

- NAT changes it to:
203.0.113.1 → 8.8.8.8

- Response comes back to public IP

- NAT forwards it to 192.168.1.2

## Advantages

- Saves IP addresses

- Improves security

- Allows multiple devices to share one public IP

Q10. Compare IPv4 and IPv6.

Ans:

| Parameter | IPv4 | IPv6 |
| --- | --- | --- |
| Address Size | 32-bit | 128-bit |
| Address Format | Decimal (e.g., 192.168.1.1) | Hexadecimal (e.g., 2001:0db8::1) |
| Address Space | Limited (~4.3 billion) | Very large (~3.4 × 10³⁸) |
| Header Size | Variable (20–60 bytes) | Fixed (40 bytes) |
| Configuration | Manual or DHCP | Auto-configuration supported |
| Security | Optional (IPSec not mandatory) | Built-in IPSec support |
| Broadcast | Supported | Not supported (uses multicast/anycast) |
| Fragmentation | By sender and routers | Only by sender |
| Address Classes | Uses classful addressing (A, B, C) | No classes (classless only) |
| Efficiency | Less efficient | More efficient and faster routing |

Q11. Analyze IPv4 datagram structure.

Ans:

- Version (4 bits)

  - Indicates IP version (IPv4 = 4)

- IHL – Internet Header Length (4 bits)

  - Specifies header length (in 32-bit words)

- Type of Service (8 bits)

  - Defines priority and quality of service

- Total Length (16 bits)

  - Total size of datagram (header + data)

- Identification (16 bits)

  - Identifies fragments of a datagram

- Flags (3 bits)

  - Controls fragmentation

  - Example: DF (Don’t Fragment), MF (More Fragments)

- Fragment Offset (13 bits)

  - Indicates position of fragment

- Time To Live – TTL (8 bits)

  - Limits packet lifetime (decreases at each router)

- Protocol (8 bits)

  - Specifies upper-layer protocol (TCP, UDP, etc.)

- Header Checksum (16 bits)

- Detects errors in header

- Source Address (32 bits)

- Sender’s IP address

- Destination Address (32 bits)

- Receiver’s IP address

- Options (Variable)

- Optional features (security, routing, etc.)

- Data (Payload)

- Actual user data

Q12. Differentiate subnetting and supernetting.

Ans:

| Parameter | Subnetting | Supernetting |
| --- | --- | --- |
| Definition | Dividing a large network into smaller sub-networks | Combining multiple networks into a larger network |
| Purpose | Improve network management and reduce congestion | Reduce routing table size and simplify routing |
| Address Usage | Efficient use within a network | Efficient use across multiple networks |
| Subnet Mask | Increases (more bits for network part) | Decreases (fewer bits for network part) |
| Network Size | Smaller networks | Larger aggregated network |
| Technique | Borrow bits from host portion | Combine network prefixes (CIDR) |
| Example | 192.168.1.0/24 → /26 (split into subnets) | 192.168.0.0/24 + 192.168.1.0/24 → /23 |

Q13. Evaluate advantages and limitations of NAT.

## Ans: Advantages of NAT

- Conserves IP Addresses

  - Allows many devices to share a single public IP

- Enhances Security

  - Hides internal IP addresses from external networks

- Cost Effective

  - Reduces the need for multiple public IPs

- Flexibility in Network Design

  - Internal addressing can be changed without affecting external communication

- Supports Private Networks

  - Enables use of private IP ranges (e.g., 192.168.x.x)

## Limitations of NAT

- Performance Overhead

  - Translation process may slow down communication

- Breaks End-to-End Connectivity

  - Makes direct device-to-device communication difficult

- Issues with Some Protocols

  - Protocols like FTP, VoIP may face problems

- Port Exhaustion (in PAT)

  - Limited number of available ports

- Complex Configuration

  - Troubleshooting and management can be difficult

Q14. Justify need of IPv6 over IPv4.

Ans: IPv4 has been widely used for Internet communication, but due to its limitations, IPv6 was developed to meet modern networking requirements.

Need of IPV6 over IPV4 are:

### 1. Address Exhaustion

- IPv4 provides about 4.3 billion addresses, which are insufficient

- IPv6 provides a vast address space (128-bit)

### 2. Efficient Addressing

- IPv4 uses classful addressing → causes wastage

- IPv6 uses classless addressing (CIDR) → efficient utilization

### 3. Better Security

- IPv4 security is optional

- IPv6 has built-in IPSec support for secure communication

### 4. Improved Performance

- Simplified header structure

- Faster and more efficient routing

### 5. No Need for NAT

- IPv6 provides sufficient addresses → eliminates need for NAT

- Ensures end-to-end connectivity

### 6. Auto Configuration

- IPv6 supports stateless auto-configuration (SLAAC)

- Devices can configure themselves automatically

### 7. Better Support for Modern Applications

- Supports IoT, mobile networks, and real-time applications

- Improved multicast and anycast

Q15. Design IPv6 addressing scheme.

Ans: An IPv6 addressing scheme defines how 128-bit IP addresses are structured and allocated to uniquely identify devices in a network.

## IPv6 Address Format

- IPv6 address = 128 bits

- Written in hexadecimal

- Divided into 8 groups (hextets) of 16 bits each

## IPv6 Address Structure

An IPv6 address is divided into two main parts:

| Part | Description |
| --- | --- |
| Network Prefix | Identifies the network |
| Interface ID | Identifies the host/device |

General Design (Typical /64 Scheme)

| Field | Bits | Purpose |
| --- | --- | --- |
| Global Routing Prefix | 48 bits | Assigned by ISP |
| Subnet ID | 16 bits | Used to create subnets |
| Interface ID | 64 bits | Identifies host |

## Types of IPv6 Addresses

- Unicast Address

  - One-to-one communication

  - Example: Global unicast

- Multicast Address

  - One-to-many communication

- Anycast Address

  - One-to-nearest communication

### Subnetting

- Add subnet ID → /64

### Host Address

- Device → 2001:0db8:1234:0001::1

## Advantages of IPv6 Addressing Scheme

- Large address space

- Efficient routing

- Hierarchical structure

- Supports auto-configuration

UNIT 4: Network Layer Routing Protocols

Q1. Define routing and state its purpose.

Ans: Routing is the process of selecting the best path for forwarding data packets from a source to a destination across one or more networks.
It is performed by routers at the Network Layer (Layer 3) of the OSI model.

## Purpose of Routing

- Path Determination

  - Finds the most efficient route for data transmission

- Packet Forwarding

  - Ensures packets reach the correct destination network

- Efficient Network Utilization

  - Optimizes use of bandwidth and network resources

- Inter-network Communication

  - Enables communication between different networks (LANs, WANs)

- Congestion Control

  - Helps avoid overloaded paths by selecting alternate routes

- Reliability

  - Provides alternate paths in case of network failure

Q2. What is the Optimality Principle?

Ans: The Optimality Principle states that:
👉 If a router lies on the optimal (best) path from source to destination, then the sub-path from that router to the destination is also optimal.

## Explanation

- In networking, routing aims to find the shortest or best path between two nodes

- According to this principle, an optimal path is made up of optimal sub-paths

- This property is used in routing algorithms like Dijkstra’s shortest path algorithm

## Example

- Suppose the best path from A → D is:
A → B → C → D

- Then:

  - Path from B → D (B → C → D) is also optimal

  - Path from C → D is also optimal

Q3. List types of routing protocols.

Ans: Routing protocols are algorithms used by routers to determine the best path for data transmission across networks.

## Types of Routing Protocols

### 1. Distance Vector Routing

- Based on distance (hop count) to destination

- Routers share routing tables with neighbors

Examples:

- RIP (Routing Information Protocol)

- IGRP

### 2. Link State Routing

- Uses complete network topology information

- Routers build a map and calculate shortest path

Examples:

- OSPF (Open Shortest Path First)

- IS-IS

### 3. Path Vector Routing

- Maintains the entire path information to destination

- Prevents routing loops

Example:

- BGP (Border Gateway Protocol)

### 4. Hybrid Routing

- Combines features of distance vector and link state

Example:

- EIGRP (Enhanced Interior Gateway Routing Protocol)

Q4. Explain static vs dynamic routing.

## Ans: 1. Static Routing

Static routing is a routing method in which routes are manually configured by the network administrator.

### Features

- Fixed routes

- No automatic updates

- Requires manual configuration

### Advantages

- Simple and easy to implement

- No routing overhead

- More secure (no external updates)

### Disadvantages

- Not scalable for large networks

- Requires manual maintenance

- Cannot adapt to network changes

## 2. Dynamic Routing

Dynamic routing is a routing method where routers automatically update routes using routing protocols.

### Features

- Automatically updates routing tables

- Adapts to network changes

- Uses routing protocols (RIP, OSPF, BGP)

### Advantages

- Scalable for large networks

- Automatically adjusts to failures

- Reduces manual work

### Disadvantages

- More complex

- Uses more bandwidth and resources

- Less secure compared to static routing

Q5. Explain intra-domain and inter-domain routing.

## Ans: 1. Intra-domain Routing

Intra-domain routing (also called Interior Routing) is the routing of data within a single autonomous system (AS).

### Explanation

- An Autonomous System (AS) is a network under a single administrative control

- Routing is done inside the same organization/network

### Features

- Faster and more efficient

- Uses internal routing protocols

- Focuses on shortest path and performance

### Examples of Protocols

- RIP (Routing Information Protocol)

- OSPF (Open Shortest Path First)

- EIGRP

## 2. Inter-domain Routing

Inter-domain routing (also called Exterior Routing) is the routing of data between different autonomous systems.

### Explanation

- Used for communication between different organizations or networks

- Requires policy-based routing decisions

### Features

- More complex

- Focuses on policies and rules

- Ensures scalability across large networks

### Example of Protocol

- BGP (Border Gateway Protocol)

Q6. Describe shortest path routing.

Ans: Shortest path routing is a routing technique in which the path between source and destination is selected based on the minimum cost (shortest path).
The cost can be measured in terms of distance, number of hops, delay, or bandwidth.

## Concept

- The network is represented as a graph

  - Nodes → Routers

  - Edges → Links

- Each link has a cost/weight

- The goal is to find the path with the minimum total cost

## Working

- Represent the network as a graph

- Assign cost to each link

- Use an algorithm to calculate the shortest path

- Update routing table with best path

## Example

Suppose:

- A → B (cost = 2)

- A → C (cost = 5)

- B → C (cost = 1)

👉 Shortest path from A to C:
A → B → C (cost = 3) instead of direct path (cost = 5)

## Advantages

- Efficient routing

- Reduces delay

- Optimizes network performance

## Disadvantages

- Requires computation

- Needs updated network information

- Complex for large networks

Q7. Apply flooding algorithm with example.

Ans: Flooding is a routing technique in which each incoming packet is sent out on all outgoing links except the one on which it arrived.
It ensures that the packet reaches the destination through all possible paths.

## Working of Flooding

- A node receives a packet

- It forwards the packet to all neighbors (except incoming link)

- Each neighbor repeats the same process

- Packet eventually reaches the destination via multiple paths

## Control Mechanisms (to avoid infinite loops)

- Hop Count (TTL) → Packet discarded after limit

- Sequence Number → Duplicate packets ignored

- Selective Flooding → Send only on selected links

## Example

Consider network:

A
     / \
    B   C
     \ /
      D

### Step-by-step (A sends to D):

- A sends packet → B and C

- B forwards → D

- C forwards → D

- D receives multiple copies of same packet

## Characteristics

- No routing table required

- Guarantees delivery (if path exists)

- Simple but inefficient

## Advantages

- Very simple

- High reliability

- No need for route discovery

## Disadvantages

- Duplicate packets

- Wastes bandwidth

- Can cause network congestion

Q8. Explain distance vector routing with example.

Ans:  Distance Vector Routing is a routing algorithm in which each router maintains a routing table (distance vector) that contains the distance (cost) and next hop to reach every destination in the network.

## Concept

- Each router knows only:

  - Distance to its neighbors

  - Information received from neighbors

- Routers periodically share their routing tables with neighbors

- Based on updates, routes are improved using Bellman-Ford algorithm

## Working

- Each router initializes its table with direct neighbors

- Routers send their routing tables to neighbors

- On receiving an update, router:

  - Compares costs

  - Updates route if a shorter path is found

- Process continues until all routers have consistent routing tables

## Example

Consider network:

A ----1---- B ----1---- C
 \                        |
  \------4---------------|

### Step 1: Initial State

- A knows: B = 1, C = 4

- B knows: A = 1, C = 1

- C knows: B = 1, A = 4

### Step 2: Update

- A receives from B → C = 1
👉 New path A → B → C = 1 + 1 = 2 (better than 4)

Final Routing Table at A

| Destination | Cost | Next Hop |
| --- | --- | --- |
| B | 1 | B |
| C | 2 | B |

## Features

- Uses distance (cost) and direction (next hop)

- Based on neighbor-to-neighbor communication

- Simple and widely used

## Advantages

- Simple implementation

- Requires less computation

- Suitable for small networks

## Disadvantages

- Slow convergence

- Routing loops may occur

- Count-to-infinity problem

Q9. Explain link state routing with example.

Ans: Link State Routing is a routing technique in which each router has complete knowledge of the network topology and calculates the shortest path using an algorithm like Dijkstra’s algorithm.

## Working

- Each router discovers its neighbors and link costs

- Creates a Link State Packet (LSP) containing this information

- Floods LSPs to all routers in the network

- Every router builds a complete network map (topology)

- Uses Dijkstra’s algorithm to find shortest paths

- Updates routing table accordingly

## Example

Consider network:

A ----1---- B ----1---- C
 \                        |
  \------4---------------|

### Process

- All routers share link information

- Each router builds same network map

- From A:

  - A → B = 1

  - A → B → C = 2 (shortest path)

👉 So best route from A to C is via B

## Features

- Uses global network information

- Fast convergence

- No routing loops

## Advantages

- Accurate routing decisions

- Fast convergence

- No count-to-infinity problem

## Disadvantages

- More complex

- Requires more memory and processing

- Higher overhead (LSP flooding)

Q10. Compare distance vector and link state routing.

Ans:

| Parameter | Distance Vector Routing | Link State Routing |
| --- | --- | --- |
| Knowledge | Knows only neighbors | Knows complete network topology |
| Algorithm | Bellman-Ford | Dijkstra |
| Information Sharing | Shares routing table with neighbors | Shares link state info with all routers |
| Convergence | Slow | Fast |
| Routing Loops | Possible | Rare/No loops |
| Complexity | Simple | Complex |
| Overhead | Low | High |
| Example Protocols | RIP | OSPF |

Q11. Analyze path vector routing.

Ans: Path Vector Routing is a routing method in which each route contains the entire path (sequence of Autonomous Systems – AS) that a packet must traverse to reach the destination.

## Concept

- Used for inter-domain routing (between different networks/AS)

- Maintains a path vector instead of just distance

- Helps avoid routing loops by checking AS paths

## Working

- Each router maintains a table with:

  - Destination

  - Path (list of AS numbers)

- Routers share path information with neighbors

- When receiving an update:

  - Router checks if its own AS is already in path

  - If yes → rejects (loop prevention)

- Selects best path based on policies and shortest AS path

## Example

- Route from A to D:
A → B → C → D

- Path vector: [A, B, C, D]

👉 If router B sees its own AS in path → it ignores that route

## Features

- Maintains complete path information

- Prevents routing loops

- Supports policy-based routing

## Advantages

- Avoids routing loops

- Suitable for large-scale networks (Internet)

- Supports flexible routing policies

## Disadvantages

- Complex implementation

- Large routing tables

- Slower convergence

Q12. Differentiate RIP, OSPF, and EIGRP.

Ans:

| Parameter | RIP | OSPF | EIGRP |
| --- | --- | --- | --- |
| Type | Distance Vector | Link State | Hybrid |
| Algorithm | Bellman-Ford | Dijkstra | DUAL |
| Metric | Hop count | Cost (bandwidth) | Bandwidth, delay, etc. |
| Max Hop Count | 15 | No limit | No fixed limit |
| Convergence | Slow | Fast | Very fast |
| Scalability | Small networks | Large networks | Medium to large networks |
| Complexity | Simple | Complex | Moderate |
| Update Type | Periodic | Triggered | Partial updates |
| Example Use | Small LANs | Enterprise networks | Cisco networks |

Q13. Evaluate advantages of OSPF over RIP.

Ans:

- Faster Convergence

  - OSPF updates routes quickly after network changes

  - RIP is slow due to periodic updates

- No Hop Count Limitation

  - OSPF has no fixed hop limit

  - RIP is limited to 15 hops

- Better Routing Metric

  - OSPF uses cost (based on bandwidth)

  - RIP uses only hop count (less efficient)

- Supports Large Networks

  - OSPF is scalable using area-based hierarchy

  - RIP is suitable only for small networks

- Efficient Updates

  - OSPF sends triggered updates only when changes occur

  - RIP sends periodic updates (wastes bandwidth)

- Loop-Free Routing

  - OSPF uses Dijkstra algorithm → avoids loops

  - RIP may suffer from routing loops

Q14. Justify the need for BGP.

Ans: BGP (Border Gateway Protocol) is a path vector routing protocol used for inter-domain routing between different Autonomous Systems (AS).

## Need for BGP

- Inter-domain Communication

  - Enables routing between different networks (AS)

- Scalability

  - Supports large-scale networks like the Internet

- Loop Prevention

  - Uses AS path to avoid routing loops

- Policy-Based Routing

  - Allows organizations to define routing policies

- Reliable Internet Routing

  - Forms the backbone of global Internet routing

Q15. Design a routing strategy for a large network

### Ans: 1. Use Hierarchical Routing

- Divide network into areas or regions

- Reduces routing complexity

### 2. Use Link-State Protocol (OSPF)

- Provides fast convergence

- Suitable for large networks

### 3. Use BGP for Inter-domain Routing

- Connects different networks or ISPs

### 4. Implement Load Balancing

- Distribute traffic across multiple paths

- Improves performance

### 5. Ensure Redundancy

- Provide backup paths

- Avoid single point of failure

### 6. Use CIDR and Subnetting

- Efficient IP address utilization

- Reduces routing table size

### 7. Apply Security Measures

- Use filtering, authentication

- Protect routing information

UNIT 5: Transport Layer

Q1. Define Transport Layer and its services.

Ans: The Transport Layer is the fourth layer (Layer 4) of the OSI model responsible for end-to-end communication between source and destination processes.
It ensures reliable and efficient delivery of data.

## Services of Transport Layer

- Process-to-Process Delivery

  - Ensures data reaches the correct application using port numbers

- Segmentation and Reassembly

  - Divides data into segments and reassembles at receiver

- Connection Control

  - Supports:

    - Connection-oriented (TCP)

    - Connectionless (UDP)

- Flow Control

  - Controls data rate between sender and receiver

- Error Control

  - Detects and corrects errors

  - Ensures reliable delivery

- Multiplexing and Demultiplexing

  - Multiple applications use same network

  - Uses port numbers to distinguish data

Q2. List duties of Transport Layer.

Ans: 1. End-to-End Delivery

  - Ensures data is delivered from source process to destination process

- Service Point Addressing (Port Addressing)

  - Uses port numbers to identify applications

- Segmentation and Reassembly

  - Breaks data into segments and reassembles them

- Flow Control

  - Prevents receiver overload

- Error Control

  - Ensures reliable data transmission

- Connection Establishment and Termination

  - Establishes, maintains, and terminates connections

- Multiplexing

  - Allows multiple processes to share the network

Q3. What is TCP? State its features.

Ans: TCP (Transmission Control Protocol) is a connection-oriented transport layer protocol that provides reliable, ordered, and error-free communication between devices over a network.

## Features of TCP

- Connection-Oriented

  - Establishes connection before data transfer (3-way handshake)

- Reliable Communication

  - Ensures delivery using ACKs and retransmissions

- Ordered Delivery

  - Data is received in the same order as sent

- Error Detection and Correction

  - Uses checksum and retransmission

- Flow Control

  - Uses sliding window mechanism

- Congestion Control

  - Adjusts transmission rate based on network conditions

- Full-Duplex Communication

  - Data can be sent and received simultaneously

Q4. Explain TCP header fields.

Ans:

- Source Port (16 bits)

  - Identifies sender application

- Destination Port (16 bits)

  - Identifies receiver application

- Sequence Number (32 bits)

  - Indicates position of data

- Acknowledgment Number (32 bits)

  - Confirms received data

- Header Length (Data Offset)

  - Specifies header size

- Flags (Control Bits)

  - SYN, ACK, FIN, RST, PSH, URG

- Window Size

  - Used for flow control

- Checksum

  - Error detection

- Urgent Pointer

  - Points to urgent data

- Options (Optional)

- Additional features

- Data (Payload)

- Actual message

Q5. Explain TCP connection establishment (3-way handshake).

Ans: TCP uses a 3-way handshake mechanism to establish a reliable connection between sender and receiver before data transmission.

## Steps of 3-Way Handshake

### Step 1: SYN (Synchronization)

- Client sends a SYN segment to server

- Contains initial sequence number (x)

### Step 2: SYN + ACK

- Server responds with SYN + ACK

- Acknowledges client’s request

- Sends its own sequence number (y)

### Step 3: ACK

- Client sends ACK to confirm connection

## Features

- Ensures both sides are ready

- Synchronizes sequence numbers

- Prevents duplicate connections

Q6. Describe flow control in TCP.

Ans: Flow control in TCP ensures that the sender does not send data faster than the receiver can handle, preventing buffer overflow.

## Mechanism: Sliding Window

### Working

- Receiver specifies a window size (buffer capacity)

- Sender can send data within that window

- As data is acknowledged → window slides forward

- If receiver buffer is full → window size becomes zero (sender stops)

## Example

- Window size = 3
👉 Sender can send 3 segments before waiting for ACK

## Advantages

- Prevents receiver overload

- Efficient data transmission

- Adapts to receiver capacity

Q7. Explain Leaky Bucket algorithm.

Ans: The Leaky Bucket Algorithm is a traffic shaping and congestion control technique that ensures data is transmitted at a constant rate, regardless of bursty input traffic.

## Working

- Incoming packets are placed in a bucket (queue)

- The bucket leaks (sends data) at a fixed rate

- If the bucket becomes full → excess packets are discarded

- Output rate remains constant and smooth

## Characteristics

- Smooths bursty traffic

- Maintains fixed output rate

- Does not allow bursts

## Example

- Bucket capacity = 10 packets

- Output rate = 1 packet/sec

## Advantages

- Prevents congestion

- Simple to implement

- Produces uniform traffic

## Disadvantages

- Discards excess packets

- Cannot handle burst traffic efficiently

Q8. Explain Token Bucket algorithm.

Ans: The Token Bucket Algorithm is a traffic control mechanism that allows data transmission at variable rates, supporting burst traffic.

## Working

- Tokens are generated at a fixed rate and stored in a bucket

- To send a packet, a token is required

- If tokens are available → packet is sent

- If no tokens → packet waits or is dropped

- Unused tokens accumulate, allowing burst transmission

## Characteristics

- Allows burst traffic

- More flexible than Leaky Bucket

- Maintains average rate

## Example

- Token rate = 1 token/sec

- Bucket size = 5 tokens

## Advantages

- Supports bursty traffic

- Efficient bandwidth usage

- Flexible transmission

## Disadvantages

- More complex than Leaky Bucket

- Requires token management

Q9. Apply congestion control using load shedding.

Ans: Load shedding is a congestion control technique in which the network intentionally discards some packets when it becomes overloaded, to maintain overall performance.

## Concept

- When network traffic exceeds capacity → congestion occurs

- Instead of slowing all traffic, the system drops selected packets

- Helps reduce load and prevent network collapse

## Working

- Router monitors traffic load

- If congestion is detected → buffer becomes full

- Router drops incoming or low-priority packets

- Remaining packets are processed efficiently

## Techniques of Load Shedding

- Random Drop

  - Packets are dropped randomly

- Priority-Based Drop

  - Low-priority packets are dropped first

- Early Drop (RED - Random Early Detection)

  - Packets are dropped before buffer becomes full

## Example

- Router buffer capacity = 100 packets

- Incoming packets = 150

## Advantages

- Prevents network collapse

- Improves overall performance

- Simple to implement

## Disadvantages

- Data loss occurs

- May affect important packets

- Not suitable for sensitive applications

Q10. Compare TCP and UDP.

Ans:

| Parameter | TCP (Transmission Control Protocol) | UDP (User Datagram Protocol) |
| --- | --- | --- |
| Type | Connection-oriented | Connectionless |
| Reliability | Reliable (ACK, retransmission) | Unreliable |
| Data Delivery | Ordered delivery | No guarantee of order |
| Error Control | Yes | Minimal |
| Flow Control | Yes | No |
| Congestion Control | Yes | No |
| Speed | Slower | Faster |
| Header Size | Larger | Smaller |
| Applications | Web, Email, File transfer | Video streaming, gaming, DNS |

Q11. Analyze TCP timers.

Ans: TCP timers are mechanisms used by TCP to ensure reliable data transmission, proper timing, and connection management.

## Types of TCP Timers

### Retransmission Timer

- Used to detect lost packets

- If ACK is not received within time → retransmit segment

### 2. Persistence Timer

- Prevents deadlock situation when receiver window is zero

- Sender periodically checks if receiver is ready

### 3. Keepalive Timer

- Checks whether an idle connection is still active

- If no response → connection is terminated

### 4. Time-Wait Timer

- Ensures delayed packets are not misinterpreted

- Waits before fully closing connection

Q12. Differentiate Leaky Bucket and Token Bucket.

ANs:

| Parameter | Leaky Bucket Algorithm | Token Bucket Algorithm |
| --- | --- | --- |
| Output Rate | Constant | Variable |
| Burst Handling | Not allowed | Allowed |
| Working Principle | Packets are sent at fixed rate | Tokens control transmission |
| Packet Loss | High (drops excess packets) | Low (buffers allowed) |
| Flexibility | Low | High |
| Traffic Type | Smooth traffic | Bursty traffic |

Q13. Evaluate advantages of UDP.

Ans: UDP (User Datagram Protocol) is a connectionless transport layer protocol that provides fast data transmission without reliability guarantees.

## Advantages of UDP

- High Speed

  - No connection setup → faster transmission

- Low Overhead

  - Small header (8 bytes) → less processing

- No Delay

  - No acknowledgment or retransmission

- Suitable for Real-Time Applications

  - Used in video streaming, VoIP, gaming

- Supports Multicasting

  - Efficient one-to-many communication

Q14. Justify use of sockets in networking.

Ans:  A socket is an endpoint for communication between two devices in a network, identified by IP address + port number.

## Need / Justification

- Process-to-Process Communication

  - Enables communication between applications

- Standard Interface

  - Provides a common API for network communication

- Supports TCP and UDP

  - Can be used for both reliable and fast communication

- Port Identification

  - Uses port numbers to identify services

- Flexibility

  - Used in client-server and peer-to-peer models

Q15. Design client-server communication using TCP sockets.

Ans: In TCP socket communication:

- Server waits for connection

- Client initiates connection

- Data is exchanged after connection establishment

## Steps Involved

### Server Side

- Create socket

- Bind socket to IP and port

- Listen for incoming connections

- Accept client connection

- Receive and send data

- Close connection

### Client Side

- Create socket

- Connect to server

- Send request

- Receive response

- Close connection

Client                          Server

| ---- Request ------------->  |

|                                             |

| <---- Response ------------ |

|                                             |

Connection established via TCP

## Features

- Reliable communication

- Uses TCP protocol

- Ensures ordered delivery

UNIT 6: Application layer

Q.1) Explain the Client–Server Paradigm in computer networks. Give a suitable example.

Ans: The Client–Server Paradigm is a network model in which:

- A client requests services

- A server provides services

Communication follows a request–response mechanism.

## Working

- Client sends a request to the server

- Server processes the request

- Server sends a response back to the client

Diagram

Client  -------- Request -------->  Server

Client  <------- Response -------  Server

## Features

- Centralized control (server manages resources)

- Multiple clients can connect to one server

- Efficient resource sharing

## Example

- Client: Web browser (Chrome)

- Server: Web server (Google, Amazon)

- Process:

  - Browser requests webpage

  - Server sends webpage as response

## Advantages

- Centralized management

- Easy data access and security control

- Scalable

## Disadvantages

- Server failure affects entire system

- High cost for server setup

- Network congestion possible

Q.2) Differentiate between TCP and UDP in terms of reliability, connection type, and applications.

Ans:

| Parameter | TCP (Transmission Control Protocol) | UDP (User Datagram Protocol) |
| --- | --- | --- |
| Reliability | Reliable (ACK, retransmission) | Unreliable (no guarantee) |
| Connection Type | Connection-oriented | Connectionless |
| Data Delivery | Ordered and error-free | No guarantee of order |
| Flow Control | Yes | No |
| Congestion Control | Yes | No |
| Speed | Slower | Faster |
| Header Size | Larger | Smaller |
| Applications | Web, Email, File transfer | Video streaming, gaming, DNS |

Q.3) Explain how communication takes place between client and server using TCP

Ans: TCP-based communication follows a connection-oriented, reliable process between a client and a server using sockets.

## Steps of Communication

### 1. Server Preparation

- Server creates a socket

- Binds it to IP address and port number

- Starts listening for connections

### 2. Client Request

- Client creates a socket

- Sends a connection request to server (connect)

### 3. Connection Establishment

- TCP performs 3-way handshake

  - SYN → SYN-ACK → ACK

### 4. Data Transmission

- Client sends request data

- Server processes and sends response data

- TCP ensures:

  - Reliable delivery

  - Ordered transmission

### 5. Connection Termination

- Connection closed using FIN and ACK signals

Diagram

Client                         Server

| ---- SYN ---------------> |

| <--- SYN-ACK ------------ |

| ---- ACK ---------------> |

| ---- Data (Request) ----> |

| <--- Data (Response) ---- |

| ---- FIN ---------------> |

| <--- ACK ---------------- |

## Features

- Reliable communication

- Error checking and retransmission

- Flow and congestion control

Q.4) Describe the Peer-to-Peer (P2P) Paradigm. How is it different from the Client–Server model?

Ans: In a Peer-to-Peer (P2P) network, all nodes (peers) are equal and can act as both client and server.

## Working

- Each peer can request and provide services

- No central server is required

## Features

- Decentralized network

- Direct communication between peers

- Resource sharing among nodes

Difference between P2P and Client–Server

| Parameter | Client–Server Model | Peer-to-Peer Model |
| --- | --- | --- |
| Structure | Centralized | Decentralized |
| Roles | Client and Server are separate | All nodes are equal |
| Control | Central server controls | No central control |
| Scalability | Limited by server | Highly scalable |
| Reliability | Server failure affects system | More robust |
| Examples | Web services | File sharing |

Q.5) What is the Application Layer in the TCP/IP model? Explain its main functions.

ANs: The Application Layer is the topmost layer of the TCP/IP model that provides network services directly to user applications.
It enables communication between software applications and the network.

## Functions of Application Layer

- Network Services to Applications

  - Provides services like web browsing, email, file transfer

- Data Representation and Formatting

  - Ensures data is in a readable format

- Process-to-Process Communication

  - Enables communication between applications

- Authentication and Security

  - Verifies users and protects data

- Resource Sharing

  - Allows sharing of files, printers, and services

- Protocol Support

  - Uses protocols like:

    - HTTP (Web)

    - FTP (File Transfer)

    - SMTP (Email)

    - DNS

Q.6) Explain the Domain Name System (DNS). How does DNS resolve a domain name into an IP address.

Ans: The Domain Name System (DNS) is a system that translates human-readable domain names into IP addresses.

## Need for DNS

- Humans remember names easily

- Computers communicate using IP addresses

## Working of DNS (Name Resolution Process)

- User enters a domain name in browser

- Request goes to local DNS resolver

- If not found → query goes to:

  - Root DNS server

  - Top-Level Domain (TLD) server (.com, .org)

  - Authoritative DNS server

- Server returns the IP address

- Browser uses IP to access the website

## Simple Flow

User → Local DNS → Root → TLD → Authoritative DNS → IP Address

## Example

- Domain: www.google.com

- DNS resolves it to: 142.250.x.x

## Features

- Hierarchical system

- Distributed database

- Fast and scalable

Q.7) Describe the File Transfer Protocol (FTP) and explain its working.

Ans:  FTP (File Transfer Protocol) is an application layer protocol used to transfer files between a client and a server over a network.

## Features

- Supports file upload and download

- Uses two connections:

  - Control connection

  - Data connection

- Works on TCP protocol

## Working of FTP

- Client establishes a control connection to server (port 21)

- Client sends commands (login, file request)

- Server responds with status messages

- A separate data connection is created (port 20)

- File transfer takes place

- Connection is closed after transfer

## Modes of FTP

- Active Mode

- Passive Mode

## Advantages

- Reliable file transfer

- Supports large files

- Provides authentication

## Disadvantages

- Less secure (data sent in plain text)

- Requires multiple connections

Q.8) Explain the Hypertext Transfer Protocol (HTTP) and its role in web communication.

Ans: HTTP (Hypertext Transfer Protocol) is an application layer protocol used for communication between web browsers and web servers.

## Working of HTTP

- User enters URL in browser

- Browser sends HTTP request to server

- Server processes request

- Server sends HTTP response (webpage)

- Browser displays content

## HTTP Message Types

- Request Message

- Response Message

## Features

- Stateless protocol

- Uses TCP (port 80)

- Simple and fast

## Role in Web Communication

- Transfers web pages (HTML, CSS, JS)

- Enables client-server communication

- Forms the foundation of the World Wide Web

## Advantages

- Simple and efficient

- Widely used

- Supports multimedia content

## Disadvantages

- Stateless (no memory of previous requests)

- Less secure (HTTP vs HTTPS)

Q.9) Describe the Simple Mail Transfer Protocol (SMTP) and explain how emails are sent using SMTP.

Ans: SMTP (Simple Mail Transfer Protocol) is an application layer protocol used for sending emails from a client to a mail server and between mail servers.

## Working of SMTP (Email Sending Process)

- User composes an email in a mail client (e.g., Gmail, Outlook)

- Mail client sends email to SMTP server

- SMTP server checks recipient address

- Email is forwarded to the recipient’s mail server

- Email is stored in recipient’s mailbox

- Receiver retrieves email using POP or IMAP

## Flow of Email

Sender → SMTP Server → Internet → Receiver Mail Server → Mailbox

## Features

- Works on TCP (port 25 / 587)

- Used only for sending emails

- Uses store-and-forward mechanism

## Advantages

- Reliable email delivery

- Widely supported

- Simple protocol

## Disadvantages

- No encryption (basic SMTP)

- Needs POP/IMAP for retrieval

Q.10) Differentiate between POP (Post Office Protocol) and IMAP (Internet Message Access Protocol).

Ans:

| Parameter | POP (Post Office Protocol) | IMAP (Internet Message Access Protocol) |
| --- | --- | --- |
| Purpose | Downloads emails to local device | Access emails directly on server |
| Storage | Emails stored locally | Emails stored on server |
| Access | Single device | Multiple devices |
| Synchronization | No synchronization | Full synchronization |
| Internet Requirement | Not required after download | Required for access |
| Management | Limited features | Advanced features |
| Speed | Faster | Slightly slower |

Q. 11) What is MIME (Multipurpose Internet Mail Extensions)? Why is it required in email communication?

Ans: MIME (Multipurpose Internet Mail Extensions) is a protocol that extends the format of email messages to support:

- Non-text data

- Multimedia content

- Multiple attachments

## Need for MIME

Originally, email (SMTP) supported only 7-bit ASCII text. MIME is required to:

- Support Multimedia Content

  - Images, audio, video

- Enable File Attachments

  - Documents, PDFs, etc.

- Handle Different Formats

  - Text, HTML, binary data

- Ensure Proper Encoding

  - Converts binary data into text format for transmission

## Features

- Uses Content-Type (text, image, audio, etc.)

- Uses Content-Transfer-Encoding (Base64, etc.)

- Allows multiple parts in one email

Q.12) Explain the process of sending an email using SMTP, POP/IMAP, and MIME.

### Ans:  Step 1: Compose Email

- User writes email

- MIME encodes attachments and multimedia

### Step 2: Sending via SMTP

- Email is sent from client to SMTP server

- SMTP forwards it to receiver’s mail server

### Step 3: Storage

- Email is stored in receiver’s mailbox

### Step 4: Retrieval via POP/IMAP

- POP → Downloads email to device

- IMAP → Accesses email on server

## Flow

User → SMTP → Mail Server → Mailbox → POP/IMAP → Receiver

Q.13) Compare the following Application Layer protocols based on their purpose and port numbers: DNS, HTTP, FTP, SMTP, POP, IMAP.

Ans:

| Protocol | Purpose | Port Number |
| --- | --- | --- |
| DNS | Resolves domain name to IP address | 53 |
| HTTP | Web communication (web pages) | 80 |
| FTP | File transfer | 21 (control), 20 (data) |
| SMTP | Sending emails | 25 / 587 |
| POP | Download emails | 110 |
| IMAP | Access emails on server | 143 |